Lab 3 Answer Key (Instructor Only)

| Module | ET4047 |
| --- | --- |
| Date | 27/09/2026 |
| Lab No. | Lab 3 – Answer Key |
| Author | Luke Griffin |

Model answers for every Your Turn exercise in the Lab 3 handout. Other correct solutions are possible. Any program that gives the right result, fits in the 18 memory cells and halts correctly should get full marks. Addresses assume the file is loaded as written: lines go in from address 0, and the model solutions do not use NOP padding.

## Task 1

1.1  Each instruction is 16 bits: an 8-bit opcode plus an 8-bit operand. Each memory address holds 8 bits, so an instruction uses two addresses, and the PC adds 2 after every fetch. HLT is the 4th instruction, at address 6.

1.2

| Line 3 becomes | Addressing mode | ACC |
| --- | --- | --- |
| LOD A | direct (label) | 5, the contents of address 34 |
| LOD 34 | direct (numeric address) | 5, the same cell as A |
| LOD #34 | immediate | 34, the number itself |
| LOD #A | immediate (label) | 34, the address of A used as a number |

1.3

| Address | Instruction | Opcode byte | Operand byte |
| --- | --- | --- | --- |
| 0 | LOD #5 | 10000111 | 00000101 (the number 5) |
| 4 | LOD A | 00000111 | 00100010 (address 34) |

Only the leftmost bit (the immediate flag) differs: 1 = immediate, 0 = direct. The opcode 0000111 (LOD) is the same.

1.4  Expected: A = 5, B = 9.

LOD #5      ; #5 -> ACC

STO A       ; ACC -> [A]

LOD #9      ; #9 -> ACC

STO B       ; ACC -> [B]

HLT

A:      0

B:      0

1.5  A third variable is needed because the ACC holds only one value. Expected: A = 8, B = 3.

LOD A       ; [A] -> ACC

STO TEMP    ; ACC -> [TEMP]   keep a copy of A

LOD B       ; [B] -> ACC

STO A       ; ACC -> [A]      A = old B

LOD TEMP    ; [TEMP] -> ACC

STO B       ; ACC -> [B]      B = old A

HLT

A:      3

B:      8

TEMP:   0

## Task 2

2.1  SUM = −1 (3 − 4) and ACC = −1. N = 1 because the result is negative, and Z = 0 because it is not zero. Watch for students who expect +1: the order is ACC − [B].

2.2

| Instruction | ACC after | Instruction | ACC after |
| --- | --- | --- | --- |
| LOD #4 | 4 | MUL #3 | 6 |
| MUL #5 | 20 | SUB #1 | 5 |
| ADD #3 | 23 | STO Y | 5 (Y = 5) |
| STO X | 23 (X = 23) | MUL X | 115 |
| LOD #2 | 2 | MUL #2 | 230, then STO W gives W = 230 |

2.3  By hand: (3 + 20) × (2 − 1) × 4 = 23 × 1 × 4 = 92. Expected X = 23, Y = 1, W = 92. This is the lecturer's Example_2.cpuvs.

; W = (3+4*5)*(8/4-1)*4

LOD #4      ; Find X = 3+4*5

MUL #5

ADD #3

STO X

LOD #8      ; Find Y = 8/4-1

DIV #4

SUB #1

STO Y

MUL X       ; Find X*Y

MUL #4      ; Find X*Y*4

STO W       ; Save result to W

HLT

X:      0

Y:      0

W:      0

2.4  Expected AVG = 7. With C = 11 the sum is 22, and 22 / 3 gives 7, not 7.33. DIV works only with integers, so the remainder is discarded. Please confirm this once in the simulator.

LOD A       ; [A] -> ACC

ADD B       ; ACC + [B] -> ACC

ADD C       ; ACC + [C] -> ACC   (21)

DIV #3      ; ACC / #3 -> ACC

STO AVG     ; ACC -> [AVG]

HLT

A:      4

B:      7

C:      10

AVG:    0

2.5  The immediate operand is only 8 bits (I8, −128 to 127), so 300 does not fit. Method 1: store 300 as data (16-bit) and use direct addressing:

LOD BIG     ; [BIG] -> ACC   (direct: 300 is stored as 16-bit data)

HLT

BIG:    300

Method 2: build 300 from small immediate values:

LOD #100    ; #100 -> ACC

MUL #3      ; ACC * #3 -> ACC

HLT

## Task 3

3.1  FLAG = 0. JNZ at address 4 is taken and jumps to address 12 (NOTZERO), so addresses 6, 8 and 10 are skipped (LOD #1, STO FLAG and JMP END). Afterwards Z = 0.

3.2  The zero case becomes the jump target:

LOD X       ; [X] -> ACC

CMP #0      ; ACC == 0?

JZ ISZERO   ; If yes, jump to ISZERO

LOD #0      ; ELSE: 0 -> [FLAG]

STO FLAG

JMP END

ISZERO: LOD #1      ; THEN: 1 -> [FLAG]

STO FLAG

END:    HLT

X:      0

FLAG:   0

3.3  Results: X = 7 gives SMALL = 1, X = 10 gives SMALL = 0 (10 − 10 = 0 is not negative), and X = 12 gives SMALL = 0. Check X = 10 in student work, as it is the edge case.

LOD X       ; [X] -> ACC

CMP #10     ; ACC - 10: negative when X < 10 (sets N)

JN LESS     ; If X < 10, jump to LESS

LOD #0      ; ELSE: 0 -> [SMALL]

STO SMALL

JMP END

LESS:   LOD #1      ; THEN: 1 -> [SMALL]

STO SMALL

END:    HLT

X:      7

SMALL:  0

## Task 4

4.1  RESULT = 2 and Z = 0, because CMP gives 4 − 3 = 1, which is not zero, so JNZ jumps to NOTEQ.

4.2  Without JMP END, the CPU "falls through" from the THEN block into the ELSE block. It stores 1 and then immediately overwrites it with 2, so RESULT is always 2, even when A = B. This is why IF-THEN-ELSE needs an unconditional jump.

4.3  Pseudo code: IF (X == 3) THEN Y = Y + 5 ELSE Z = Z + 2 ENDIF; X = 8.

| Starting X | Block that runs | X after | Y after | Z after |
| --- | --- | --- | --- | --- |
| 3 | THEN | 8 | 5 | 0 |
| 5 | ELSE | 8 | 0 | 2 |

X is always 8, because X = 8 comes after ENDIF. The PC is 24 at the end, matching the lecture's "after execution" screenshot.

4.4  Expected MAX = 9 in both tests. After CMP the ACC still holds A, so STO MAX stores A without reloading it. Accept solutions that reload A. If A = B, either value is correct.

LOD A       ; [A] -> ACC

CMP B       ; ACC - [B]  (ACC still holds A afterwards)

JN BBIG     ; If A < B, jump to BBIG

STO MAX     ; THEN: ACC -> [MAX]  (A is still in ACC)

JMP END

BBIG:   LOD B       ; ELSE: [B] -> ACC

STO MAX     ;       ACC -> [MAX]

END:    HLT

A:      4

B:      9

MAX:    0

## Task 5

5.1  SUB #1 runs 6 times. When JNZ jumps, the PC is loaded with 2, the address of LOOP.

5.2  With N = 0, SUB #1 makes N = −1 before the first test, so N is never 0 when CMP runs. The program counts down through negative numbers and does not halt normally. This is a REPEAT-UNTIL loop, which always runs the body at least once. The fix is the WHILE-DO pattern, which tests first:

LOD N       ; [N] -> ACC

LOOP:   CMP #0      ; WHILE N != 0   (test BEFORE decrementing)

JZ DONE     ; If N == 0, jump to DONE

SUB #1      ; ACC - 1 -> ACC

STO N       ; ACC -> [N]

JMP LOOP    ; Jump back to LOOP

DONE:   HLT

N:      0

5.3

| Pass | COUNT | SUM |
| --- | --- | --- |
| 1 | 1 | 1 |
| 2 | 2 | 3 |
| 3 | 3 | 6 |
| SUM for MAX = 5 | 5 | 15 (1 + 2 + 3 + 4 + 5) |

5.4  WHILE-DO tests at the top and must leave the loop when the condition becomes false (COUNT == MAX), so it jumps forwards with JZ ENDWHILE. The countdown tests at the bottom and must repeat while the counter is not yet zero, so it jumps backwards with JNZ LOOP.

## Task 6

6.1

| A | B | W | A | B | W |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | −1 | 0 | −1 |
| 0 | −1 | −1 | −1 | −1 | −1 |

6.2  An OR gate. By De Morgan's law, NOT(NOT A AND NOT B) = A OR B. Because the operations are bitwise, A = 5 (0101) and B = 3 (0011) give W = 0111 = 7.

6.3  Expected W = 0 for A = B = −1. Every other combination of 0 and −1 gives −1.

LOD A       ; [A] -> ACC

AND B       ; ACC AND [B] -> ACC

STO T       ; ACC -> [T]

NOT T       ; NOT([T]) -> ACC

STO W       ; ACC -> [W]

HLT

A:      -1

B:      -1

T:      0

W:      0

## Challenges

C.1  This is the lecturer's Quotient_and_Reminder.cpuvs. Expected Q = 3, R = 2.

LOD #0      ; Initialisation: 0 -> ACC

STO Q       ; 0 -> [Q]

LOD A       ; [A] -> ACC

STO R       ; [A] -> [R]

LOOP:   LOD R       ; [R] -> ACC

CMP B       ; Test R < B?

JN DONE     ; If yes, go to DONE

SUB B       ; Otherwise ACC - [B] -> ACC

STO R       ;   ACC -> [R]

LOD Q       ;   [Q] -> ACC

ADD #1      ;   ACC + 1 -> ACC

STO Q       ;   ACC -> [Q]

JMP LOOP    ; Repeat while R >= B

DONE:   HLT         ; Stop execution

A:      17

B:      5

Q:      0           ; 3 after execution

R:      0           ; 2 after execution

C.2  Expected RES = 120. If N starts at 1, the loop still runs once: RES = 1 and N becomes 0. Then CMP #1 is never zero again, so the loop never ends. Fix: test N before multiplying (WHILE-DO), or compare with 1 at the start of the loop.

LOOP:   LOD RES     ; [RES] -> ACC

MUL N       ; ACC * [N] -> ACC

STO RES     ; ACC -> [RES]

LOD N       ; [N] -> ACC

SUB #1      ; ACC - 1 -> ACC

STO N       ; ACC -> [N]

CMP #1      ; ACC == 1?

JNZ LOOP    ; If not, repeat

HLT

N:      5

RES:    1

## Review Questions

R1  LOD 34 is direct: it loads the contents of address 34, which is the same as LOD A. LOD #34 is immediate: it loads the number 34.

R2  CMP updates the flags exactly as SUB would, but leaves the ACC unchanged. It sets up the Z and N flags for the jump without destroying the value being tested.

R3  The PC. A jump overwrites the PC with the operand address, so the next fetch comes from there. Jumping to an earlier address repeats instructions, which is a loop.

R4  ADD, SUB, MUL, DIV, AND and CMP. LOD and STO do not update the flags, so after a LOD the flags still describe an older result. CMP is needed to set them for the value just loaded.

R5  The operand field of an instruction is only 8 bits, which gives the range −128 to 127 in two's complement. A data value fills a whole 16-bit memory word (I16), which gives −32768 to 32767.