Lab 3: Linear and Circular Queues

| Module | ET4047 |
| --- | --- |
| Date | 27/09/2026 |
| Lab No. | Lab 3 |
| Author | Luke Griffin |

# 1  Lab tasks

Work in your Linear_Queue_Example project unless a task says otherwise. Save a copy of the sketch after each task (for example Task3.ino, Task4.ino) so you can go back to it. Paste the Serial Monitor output into your lab report for every task marked (record).

## Task 1: Run and compare both examples

- Run both example sketches in Wokwi. (record) Paste both outputs into your report.

- The two outputs are identical. Explain in one or two sentences why this test cannot reveal any difference between the two designs.

Your answer:

## Task 2: Add a printQueue() function

Debugging a queue is much easier when you can see inside it. Add a function to the linear sketch with this signature, placed above setup():

void printQueue(Queue *q);

It should print front, rear and count, then every slot of the array. Show the value for slots that are in the queue and [--] for slots that are not. For example:

Required format

front=2 rear=4 count=3 ¦ [--] [--] [30] [40] [50]

Hint In the linear queue, slot i is part of the queue when count > 0 and front <= i <= rear. Use a for loop over all MAX_QUEUE_SIZE slots.

## Task 3: Expose the problem

- Change #define MAX_QUEUE_SIZE 10 to 5.

- Replace the body of setup() (keep Serial.begin(9600);) with the test sequence below.

- Run it. (record) Paste the output into your report.

Task 3 test sequence

initializeQueue(&myQueue); enqueue(&myQueue, 10); enqueue(&myQueue, 20); enqueue(&myQueue, 30); enqueue(&myQueue, 40); enqueue(&myQueue, 50); printQueue(&myQueue); Serial.println(); dequeue(&myQueue); dequeue(&myQueue); printQueue(&myQueue); Serial.println(); enqueue(&myQueue, 60); enqueue(&myQueue, 70); printQueue(&myQueue); Serial.println();

Answer: (a) How many elements are in the queue when 60 is rejected? (b) Which line of code causes the rejection? (c) Would the problem go away if you made MAX_QUEUE_SIZE bigger? Explain.

Your answer:

## Task 4: Fix 1, shift elements on dequeue

Modify dequeue() so that after the front element is saved, every remaining element slides one place to the left. front then always stays at 0, and the freed slot moves to the end of the array where enqueue() can reuse it.

- Write a for loop that copies items[i + 1] into items[i] for every element in the queue. Should it run forwards or backwards this time? Why?

- Remove the q->front++ line and decrement rear instead.

- Change isFull() to return q->count == MAX_QUEUE_SIZE;. This is the clearest definition of “full”.

Re-run the Task 3 test. (record) 60 and 70 should now be accepted. Your final printQueue() line should show:

Expected final line

front=0 rear=4 count=5 ¦ [30] [40] [50] [60] [70]

Think about it This works, but what does it cost? If the queue holds n elements, how many copy operations does one dequeue() now perform? Record your answer in your lab report.

## Task 5: Fix 2, convert the linear queue to a circular queue

Start from your Task 3 version (not Task 4) and turn it into a circular queue yourself. Do not copy the circular example; use it only to check your work afterwards.

- isFull(): use count instead of rear.

- enqueue(): replace rear++ with a wrap-around increment using %.

- dequeue(): replace front++ with a wrap-around increment. Is the “reset when empty” block still needed? Try it with and without.

- enqueueToOppositeEnd(): delete the shifting loop entirely. Step front backwards with wrap-around and write the value there. If the queue was empty beforehand, make rear equal front too.

- printQueue(): your Task 2 test front <= i <= rear no longer works once rear wraps behind front. Fix it using the hint below.

Hint for printQueue() Work out how far slot i is from the front, moving forwards with wrap-around: int offset = (i - q->front + MAX_QUEUE_SIZE) % MAX_QUEUE_SIZE; Slot i is part of the queue when offset < q->count.

Re-run the Task 3 test. (record) The final line should show 60 and 70 stored in slots 0 and 1:

Expected final line

front=2 rear=1 count=5 ¦ [60] [70] [30] [40] [50]

## Task 6: Stress test

A single test is not proof. Add the function below to your sketch and call it at the end of setup() (after destroyQueue(&myQueue);). It keeps one element in the queue at all times, so the “reset when empty” trick never rescues it, and then pushes 20 more values through a 5-slot queue, checking they come out in the right order.

Task 6 stress test

void stressTest(Queue *q) { int next = 1; // next value to enqueue int expected = 1; // value we expect dequeue() to return int failures = 0; // Keep one item in the queue so it never becomes empty enqueue(q, next++); for (int round = 1; round <= 10; round++) { for (int i = 0; i < 2; i++) { if (!enqueue(q, next++)) failures++; } for (int i = 0; i < 2; i++) { if (dequeue(q) != expected++) failures++; } } Serial.print("Stress test finished. Failures: "); Serial.println(failures); }

- Run the stress test on the original linear queue (Task 3 version). (record) Record the number of failures.

- Run it on your Task 4 and Task 5 versions. (record) Both must report Failures: 0.

- Look at the original version’s output. At which value does the first “Queue is full” error appear, and why at that point?

Your answer:

## Task 7 (extension): Interactive queue over Serial

Make the queue interactive using the Wokwi Serial Monitor input box. In loop(), read a line with Serial.readStringUntil('\n') and respond to these commands, calling printQueue() after each one:

| Command | Action |
| --- | --- |
| e 42 | enqueue 42 |
| f 7 | enqueueToOppositeEnd 7 |
| d | dequeue and print the value |
| p | peek and print the value |
| c | clear (destroyQueue) |

Use it to demonstrate the wrap-around live: fill the queue, dequeue a few, then enqueue again. Hint: String.toInt() converts text to a number, and Serial.available() tells you when input has arrived.